# PARALLEYE Technical Methodology

## Chapter 2 – Engineering Principles, System Boundaries and Implementation Governance

Document: PARALLEYE Technical Methodology
Chapter: 2 – Engineering Principles, System Boundaries and Implementation Governance
Version: 1.0
Status: Draft for Approval
Project: PARALLEYE Android MVP

# 1. Purpose of This Chapter

This chapter defines the engineering principles that shall govern the implementation of the PARALLEYE methodology.

Chapter 1 established the complete methodological chain:

Sensor acquisition → angle measurement → neck-flexion estimation → angle-load mapping → activity classification → activity multiplier → cumulative load → recovery → score → adaptive alert → long-term progress

This chapter defines how those components must be separated, connected, documented, configured and tested within the Android application.

The purpose is to prevent the methodology from being implemented as a single opaque algorithm or simplified into a direct angle-threshold alert.

Every major methodology component shall remain:

- Separately identifiable.

- Independently testable.

- Centrally configurable.

- Traceable to an approved requirement.

- Replaceable or adjustable without unnecessary changes to unrelated components.

# 2. Core Engineering Objective

The engineering objective is to create an Android implementation that faithfully represents the PARALLEYE methodology while remaining suitable for:

- APK testing.

- Pilot deployment.

- Parameter optimisation.

- Future research validation.

- Technical handover.

- Future cloud and institutional expansion.

- Maintenance by developers other than the original builder.

The codebase must preserve the distinction between:

- Objective measurement.

- Behavioural interpretation.

- User-facing intervention.

- Historical reporting.

These functions may exchange data, but they must not be merged into one unexplained calculation.

# 3. Source-of-Truth Hierarchy

The implementation shall follow an explicit document hierarchy.

Where requirements or parameter values conflict, the application must not silently choose or merge them.

The working priority shall be:

- Latest approved PARALLEYE written clarification.

- Approved Technical Methodology document.

- Filed invention document.

- Approved Functional Specification.

- Approved UI/UX Specification.

- Approved test and acceptance criteria.

- Provisional parameter tables.

The invention document establishes the major system elements, including sensor-based measurement, activity classification, activity-dependent weighting, cumulative load, recovery, score normalisation and adaptive feedback.

The separate algorithm specification contains provisional development parameters intended to remain configurable during pilot optimisation.

Accordingly:

- The methodology structure is authoritative.

- Numerical values may remain provisional where the documents differ.

- Every active parameter set must be explicitly identified.

- Conflicting values must be documented rather than concealed.

- Claude Code or any developer must not invent a compromise value without approval.

# 4. Fundamental Separation of System Domains

PARALLEYE shall be implemented using four principal system domains.

## 4.1 Measurement Domain

The Measurement Domain determines the objective physical and contextual state of the device.

It includes:

- Sensor acquisition.

- Sensor timestamps.

- Sensor fusion.

- Signal filtering.

- Device orientation.

- Device-angle calculation.

- Neck-flexion estimation.

- Screen orientation.

- Activity-related observations.

- Activity classification.

- Engagement-related observations.

The Measurement Domain should answer:

What is objectively occurring at this moment?

It must not decide:

- Whether the user deserves a score bonus.

- Whether the mascot should appear.

- Whether the user’s baseline should be considered an acceptable posture.

- Whether accumulated exposure should be forgiven.

- Whether a historical trend represents improvement.

## 4.2 Behaviour Domain

The Behaviour Domain interprets objective measurements across time.

It includes:

- Calibration-based behavioural personalisation.

- Angle-load mapping.

- Activity multipliers.

- Dynamic-load calculation.

- Cumulative-load accumulation.

- Stress-state classification.

- Recovery.

- Score normalisation.

- Responsiveness adjustment.

- Alert-state decision logic.

The Behaviour Domain should answer:

What does the measured and accumulated exposure mean, and what response is appropriate?

The Behaviour Domain may use the user’s baseline and behavioural history.

It must not alter the objective live sensor measurement.

## 4.3 Presentation Domain

The Presentation Domain communicates the system state to the user.

It includes:

- Current score.

- Monitoring status.

- Posture-zone display.

- Ms Angle Angel states.

- Alert animation.

- Session summaries.

- Progress views.

- Settings.

- Permission explanations.

- Error messages.

The Presentation Domain must display the outputs of the underlying methodology.

It must not independently recalculate angle, load, recovery or score.

## 4.4 Persistence and Reporting Domain

The Persistence and Reporting Domain stores and aggregates information.

It includes:

- User profile.

- Original baseline.

- Settings.

- Parameter version.

- Monitoring sessions.

- Score history.

- Zone durations.

- Activity distribution.

- Alert events.

- Dismissal events.

- Correction events.

- Recovery periods.

- Long-term trends.

This domain should preserve historical records without interfering with the current live calculation.

# 5. Direction of Data Flow

The normal flow of information shall proceed in one direction through the methodology:

Sensors

↓

Signal Processing

↓

Device Angle

↓

Neck-Flexion Estimate

↓

Angle-Load Value

↓

Activity and Engagement Context

↓

Dynamic Load Increment

↓

Cumulative Load

↓

Stress State and Score

↓

Alert Decision

↓

User-Facing Intervention

↓

Correction and Recovery

↓

Session and Long-Term Records

Feedback from later stages may affect future behavioural decisions, but it must not retrospectively alter the objective sensor reading.

Examples:

- A user dismissing Ms Angle Angel may affect responsiveness scoring.

- A successful correction may activate recovery.

- Long-term improvement may alter future behavioural progression if approved.

- None of these events may change the angle that was physically measured at that moment.

# 6. Objective Measurement Principle

The live device angle must be generated only from the approved sensor-processing methodology.

It must not be adjusted merely because:

- The user calibrated at a poor posture.

- The user is tall or short.

- The user is gaming.

- The user selected low sensitivity.

- The score is already low.

- The user recently dismissed an alert.

- The user has historically poor posture.

These factors may influence behavioural interpretation or intervention, but not the physical measurement itself.

The filed invention describes device tilt angle as a separate input that is subsequently combined with activity-based weighting and cumulative load processing.

The implementation must therefore preserve distinct values for:

- Raw sensor values.

- Filtered sensor values.

- Measured device angle.

- Estimated neck-flexion angle.

- Angle-load value.

- Dynamic-load increment.

- Cumulative-load value.

- Display score.

These values must not be stored under one ambiguous variable such as postureValue.

# 7. Calibration Boundary Principle

Calibration is part of behavioural personalisation.

It is not part of the raw sensor calculation.

Calibration may establish:

- Original posture baseline.

- Typical hold angle.

- Typical activity patterns.

- Initial behavioural goals.

- Progressive feedback strategy.

- Long-term comparison reference.

The filed invention describes a user-specific baseline based on factors such as height, typical hold angle, duration patterns, pickup frequency, orientation frequency and typing duration.

However, the engineering boundary must remain:

Calibration may affect behavioural progression, but must not redefine the objective angle.

The calibration module must therefore output a dedicated baseline profile rather than modifying the angle-calculation function.

# 8. Activity Classification Boundary Principle

Activity classification shall remain modular and independent from angle calculation.

The algorithm specification explicitly states that activity classification should be modular and independently adjustable.

The activity module shall receive permitted observations such as:

- Foreground application category.

- Keyboard-active state.

- Device movement pattern.

- Touch or interaction signals where technically and legally available.

- Device orientation.

- Continuous interaction duration.

It shall output:

- Activity category.

- Classification confidence where implemented.

- Evidence source.

- Time of classification.

- Fallback category when uncertain.

It shall not output:

- A modified angle.

- A final score.

- A direct mascot instruction.

- A cumulative-load value.

The Behaviour Domain shall apply the relevant multiplier after classification.

# 9. Unknown Activity Principle

PARALLEYE must prefer uncertainty over false certainty.

Where the system cannot reliably distinguish between activities, it shall use an approved fallback such as:

- General use.

- Unknown.

- Unclassified.

The application must not label activity as gaming, typing, scrolling or video watching without sufficient supporting evidence.

The source documents allow activity detection through multiple platform-appropriate signals rather than prescribing that every category must always be identified.

The fallback multiplier must be explicitly defined and configurable.

# 10. Non-Linear Load Principle

The implementation must not accumulate raw angle directly unless explicitly approved in a later methodology revision.

The measured or estimated angle shall first pass through an angle-load mapping layer.

The provisional mapping increases non-linearly as angle severity rises, assigning progressively greater values at higher ranges.

This separation is essential because:

- Angle is a physical estimate.

- Angle load is a behavioural or biomechanical weighting.

- Cumulative load is an exposure state.

- Score is a normalised user-facing representation.

These are related but different quantities.

# 11. Time and Frame Independence Principle

The filed methodology describes cumulative load as updating over successive measurement frames using a frame factor.

However, Android devices may:

- Produce different sensor frequencies.

- Drop or delay sensor events.

- Enter power-saving states.

- Pause background processing.

- Vary in hardware performance.

The implementation must therefore prevent the same posture duration from producing substantially different cumulative load merely because one device emits more sensor events than another.

The engineering design should maintain the conceptual frame-factor model while controlling for real elapsed time and actual sample intervals.

A robust implementation may:

- Use sensor timestamps.

- Calculate a normalised update interval.

- Scale the frame factor relative to the intended sampling rate.

- Reject duplicate or invalid timestamps.

- Pause accumulation during missing or invalid data.

The selected method must be documented.

The application must not assume that every sensor callback represents an identical duration.

# 12. State Ownership Principle

Each important state must have one authoritative owner.

Examples:

| State | Authoritative owner |
| --- | --- |
| Raw sensor values | Sensor acquisition component |
| Filtered orientation | Signal-processing component |
| Device angle | Angle-calculation component |
| Neck-flexion estimate | Neck-estimation component |
| Activity category | Activity-classification component |
| Activity multiplier | Behaviour configuration |
| Dynamic-load increment | Dynamic-load component |
| Cumulative load | Cumulative-load engine |
| Recovery state | Recovery engine |
| 0–100 score | Scoring engine |
| Mascot state | Alert-state engine |
| Baseline profile | Calibration repository |
| Session summary | Reporting component |

No two components should independently maintain competing versions of the same live state.

# 13. State Immutability and Traceability

Where practical, each processing cycle should create a traceable evaluation record containing the inputs and outputs used for that cycle.

A cycle record may include:

- Timestamp.

- Device orientation.

- Raw or filtered angle.

- Estimated neck flexion.

- Posture zone.

- Angle-load value.

- Activity category.

- Activity multiplier.

- Engagement context.

- Dynamic-load increment.

- Cumulative load before update.

- Cumulative load after update.

- Recovery amount.

- Score.

- Stress state.

- Mascot state.

This does not mean every high-frequency cycle must be permanently stored.

It means the architecture should allow:

- Debug logging.

- Test replay.

- Pilot validation.

- Investigation of unexpected behaviour.

- Comparison of expected and actual calculations.

Sensitive or high-volume logs may be disabled in release builds.

# 14. Single Calculation Path Principle

There shall be one approved calculation path for each methodology output.

For example:

- The dashboard must not calculate its own score.

- The overlay must not maintain a separate cumulative load.

- The session summary must not reclassify historical activities using different rules.

- Landscape mode must not use an unrelated scoring engine.

- The settings screen must not directly alter calculations without updating the central configuration.

All user interfaces shall consume outputs from the same domain logic.

# 15. Portrait and Landscape Consistency

Portrait and landscape operation must share the same overall methodology.

Differences may exist in:

- Axis transformation.

- Orientation interpretation.

- Filtering.

- Activity context.

- Thresholds.

- Dwell or qualification timing.

- Weighting.

- Alert placement.

- Mascot animation.

Any difference must be explicit.

The landscape implementation must not be created as a completely separate undocumented algorithm.

Both orientations shall ultimately produce the same defined output types:

- Device angle.

- Estimated neck-flexion angle.

- Angle-load value.

- Activity.

- Dynamic load.

- Cumulative load.

- Recovery.

- Score.

- Mascot state.

# 16. Monitoring-State Principle

The monitoring engine shall use clearly defined states.

Recommended states include:

- Not configured.

- Permissions required.

- Ready.

- Monitoring.

- Temporarily paused.

- Screen off.

- Sensor unavailable.

- Background restricted.

- Restart recovery.

- Stopped by user.

- Error.

The application must define whether accumulation is permitted in each state.

For example:

| Monitoring state | Sensor processing | Load accumulation | Alert display |
| --- | --- | --- | --- |
| Monitoring | Yes | Yes | Yes |
| Temporarily paused | No | No | No |
| Screen off | As approved | No unless justified | No |
| Sensor unavailable | No | No | No |
| Background restricted | Limited or no | No if unreliable | No |
| Restart recovery | Initialising | No until valid | No |
| Stopped by user | No | No | No |

The exact rules shall be finalised in later chapters.

# 17. Session Boundary Principle

A monitoring session must have a defined beginning and end.

Potential session-start conditions may include:

- Screen becomes active.

- User interaction begins.

- Monitoring service resumes.

- Valid sensor and activity data become available.

Potential session-end conditions may include:

- Screen turns off.

- Extended inactivity.

- Monitoring is paused.

- Service stops.

- Device shuts down.

- Application data is reset.

The implementation must specify:

- Whether cumulative load resets at session start.

- Whether some current load carries forward.

- Whether the live score persists.

- How interrupted sessions are handled.

- How daily summaries aggregate sessions.

- How a device restart affects the session.

These decisions must not be left to accidental process behaviour.

# 18. Current, Session and Historical Data Separation

The application shall distinguish:

## 18.1 Current data

Used for immediate behavioural decisions.

Examples:

- Current angle.

- Current activity.

- Current load.

- Current score.

- Current alert state.

## 18.2 Session data

Used to summarise a single period of phone use.

The algorithm specification requires metrics such as session duration, average score, minimum and maximum score, zone durations, alerts, dismissals and activity distribution.

## 18.3 Historical data

Used for daily, weekly, monthly and baseline comparisons.

A correction may improve current load and score without deleting:

- Previous red-zone exposure.

- Earlier alerts.

- Earlier score reductions.

- Session summaries.

- Historical trends.

# 19. Recovery-State Principle

Recovery must be implemented as a controlled state, not merely as a negative number added whenever the angle falls below a threshold.

The Recovery Engine must determine:

- Whether the posture is within the qualifying range.

- Whether the correction has lasted long enough.

- Whether sensor data is stable.

- Whether the user is actively using the phone.

- Whether recovery should begin.

- The amount of recovery.

- Whether recovery pauses or resets after deterioration.

- Whether a mascot should retreat.

- Whether the score should rise.

- Whether the event qualifies for a responsiveness reward.

The filed document defines proportional recovery below 15°, with greater recovery closer to neutral.

The separate algorithm specification includes a 120-second qualification and a fixed initial rate.

These alternatives must be represented through configuration or a selected versioned recovery strategy.

# 20. Alert-State Principle

Ms Angle Angel must be controlled by a dedicated alert-state engine.

The state engine shall determine:

- Idle.

- Peek.

- Peel.

- Full alert.

- Dismissed temporarily.

- Awaiting correction.

- Retreating.

- Cooldown.

- Suppressed because monitoring is unavailable.

The alert UI must not decide its own state from raw angle.

The progressive mascot behaviour described in the invention is driven by cumulative load or the resulting behavioural state.

A user tap shall be treated as a UI action, not proof of correction.

# 21. Dismissal and Correction Principle

The application must distinguish:

## Dismissal

The user taps Ms Angle Angel to remove the visible interruption.

## Correction

The sensor methodology confirms that posture has returned to the approved range and remained there for the required duration.

These are separate events.

Dismissal may:

- Hide the mascot temporarily.

- Start a reappearance timer.

- Record a dismissal event.

- Affect responsiveness logic if correction does not follow.

Correction may:

- Cause the mascot to retreat.

- Activate recovery.

- Improve the score.

- Qualify for positive reinforcement.

- Record a successful response.

The application must never interpret a tap alone as posture correction.

# 22. Score Integrity Principle

The 0–100 score must be calculated by one dedicated scoring engine.

The score shall be based on the approved methodology and must not be manually manipulated for visual effect.

It must define:

- Input cumulative load.

- Scaling factor.

- Recovery influence.

- Responsiveness adjustment.

- Minimum.

- Maximum.

- Rounding.

- Update frequency.

- Persistence.

- Current versus session display.

The score must be clamped after all approved adjustments.

A bonus must not allow the displayed score to exceed 100.

A penalty must not reduce it below 0.

# 23. Parameter Centralisation Principle

All adjustable methodology values must be stored in a central, version-controlled configuration.

No important value should be repeated as unexplained numbers across multiple source files.

Configurable values include:

- Sampling rate.

- Sensor-filter parameters.

- Zone boundaries.

- Angle-load values.

- Activity multipliers.

- Unknown-activity multiplier.

- Frame factor.

- Recovery threshold.

- Recovery qualification duration.

- Recovery rate.

- Scaling factor.

- Responsiveness bonus.

- Responsiveness penalty.

- Bonus limits.

- Mascot-state ranges.

- Dismissal interval.

- Reappearance interval.

- Cooldown.

- Sensitivity modifiers.

- Portrait parameters.

- Landscape parameters.

The algorithm specification expressly identifies the main methodology values as adjustable during development and pilot phases.

# 24. Parameter-Version Principle

Every test build should record the parameter version it uses.

A parameter-version record should include:

- Configuration version.

- Build version.

- Date.

- Active multiplier set.

- Active recovery strategy.

- Active alert-state ranges.

- Active scaling factor.

- Any orientation-specific values.

This enables pilot results to be interpreted accurately.

Without parameter versioning, data from two builds may appear comparable even when the underlying methodology differs.

# 25. Sensitivity-Setting Principle

Low, Medium and High sensitivity shall remain user-selectable in the MVP unless later removed through an approved change.

Sensitivity must be implemented through documented parameter adjustments.

It must not alter the raw angle calculation.

It may affect approved behavioural values such as:

- Accumulation sensitivity.

- Alert qualification time.

- Alert-state boundaries.

- Reappearance timing.

- Recovery qualification.

- Other approved intervention parameters.

The effect of each sensitivity level must be explicit and testable.

# 26. No Hidden Placeholder Principle

The application must not present placeholder methodology as a completed feature.

Where a required formula or parameter is unresolved:

- The component architecture may be implemented.

- A temporary value may be used only when clearly labelled.

- The placeholder must be listed in the open-questions document.

- The technical handover must identify it.

- The UI must not imply scientific validation where none exists.

Examples include:

- Provisional device-to-neck transformation.

- Unvalidated activity confidence thresholds.

- Provisional multiplier values.

- Provisional recovery qualification.

- Provisional alert-state ranges.

# 27. Privacy-by-Design Principle

The application shall minimise data collection.

The invention specifies operation without camera, microphone or biometric collection.

The engineering implementation shall:

- Avoid camera permissions.

- Avoid microphone permissions.

- Avoid biometric processing.

- Request only necessary Android permissions.

- Explain each permission.

- Process data locally for the MVP unless otherwise approved.

- Avoid third-party analytics unless explicitly authorised.

- Avoid collecting content typed or viewed by the user.

- Store classifications rather than private content.

For example, the application may record that the activity was “Typing” without storing what was typed.

# 28. Android Platform Compliance Principle

PARALLEYE shall operate within Android security and background-processing rules.

The implementation must not attempt to bypass platform restrictions.

Where required, the application may use:

- Foreground services.

- Notification channels.

- Usage-access permission.

- Overlay permission.

- Boot-completed receiver.

- Battery-optimisation guidance.

- Local database storage.

- Appropriate lifecycle-aware components.

Any feature requiring a sensitive permission must:

- Explain why it is needed.

- Fail safely if denied.

- Avoid pretending that monitoring remains fully functional when it does not.

- Provide a clear permission-status view.

# 29. Graceful Degradation Principle

The application should continue operating at the highest valid level when some information is unavailable.

Examples:

- If activity cannot be classified, use Unknown or General Use.

- If gyroscope data is unavailable, use the approved fallback angle method and record reduced capability.

- If usage access is denied, continue posture monitoring without app-category classification.

- If overlay permission is denied, use an approved alternative notification mechanism or clearly state that visual intervention is unavailable.

- If auto-start is restricted, inform the user that manual launch may be required.

- If local storage fails, stop claiming that history is being saved.

Graceful degradation must never mean fabricating unavailable data.

# 30. Battery and Performance Principle

PARALLEYE is intended to run passively during normal smartphone use.

The implementation must balance methodological fidelity with reasonable resource use.

It shall monitor:

- Sensor sampling load.

- CPU use.

- Memory use.

- Wake-lock behaviour.

- Foreground-service behaviour.

- Database-write frequency.

- Overlay rendering.

- Battery consumption.

High-frequency sensor readings should not automatically result in equally frequent database writes or UI updates.

The application may:

- Process sensors at the approved frequency.

- Update the live UI at a lower human-readable frequency.

- Aggregate historical data before storage.

- Batch non-critical writes.

Any optimisation must preserve the accuracy of cumulative exposure calculations.

# 31. Testability Principle

Each major component shall support independent tests.

At minimum, the following should be testable without relying on the complete UI:

- Sensor-vector-to-angle calculation.

- Portrait axis mapping.

- Landscape axis mapping.

- Angle-to-load mapping.

- Activity-to-multiplier mapping.

- Dynamic-load increment.

- Cumulative-load update.

- Recovery calculation.

- Score calculation.

- Score clamping.

- Stress-state classification.

- Mascot-state transition.

- Dismissal and reappearance.

- Sensitivity-level effects.

- Session-summary aggregation.

The implementation should support deterministic test inputs and expected outputs.

# 32. Replay and Simulation Principle

The architecture should allow prerecorded or synthetic sensor and activity sequences to be replayed through the methodology.

A test sequence may include:

- Green posture for 30 seconds.

- Yellow posture while scrolling.

- Red posture while gaming.

- Alert dismissal without correction.

- Continued red posture.

- Full alert reappearance.

- Correction to green.

- Sustained recovery.

- Score improvement.

- Session completion.

Replay testing allows the team to validate the entire methodology without manually reproducing every movement on a physical device.

# 33. Logging Principle

Development builds should support structured technical logging.

Logs may include:

- Monitoring-state changes.

- Permission status.

- Sensor availability.

- Angle output.

- Activity classification.

- Multiplier selection.

- Dynamic-load increment.

- Cumulative-load update.

- Recovery.

- Score.

- Alert-state transition.

- Session start and end.

- Errors.

Logs must not include:

- Typed content.

- Messages.

- Video content.

- Private screen content.

- Passwords.

- Personal identifiers beyond what is strictly needed.

Release logging must be reduced to protect privacy and performance.

# 34. Error-Handling Principle

Every critical component must fail predictably.

Examples:

- Invalid sensor vector: discard the sample and log the event.

- Division or square-root error: return no valid angle rather than an arbitrary number.

- Missing activity information: use approved fallback.

- Corrupted configuration: load a validated default configuration and report the problem.

- Database failure: stop writing and inform the user or developer log.

- Overlay failure: preserve monitoring but report intervention limitation.

- Service termination: recover according to Android rules where possible.

The application must not continue accumulating load using stale sensor values.

# 35. Configuration Validation Principle

Before monitoring begins, the active configuration must be validated.

Validation should confirm:

- Zone thresholds are ordered.

- Angle ranges do not unintentionally overlap.

- Load values are non-negative.

- Multipliers are valid.

- Frame factor is positive.

- Recovery cannot push cumulative load below zero.

- Score ranges cover 0–100 without unintended gaps.

- Mascot states are ordered.

- Timing values are non-negative.

- Sensitivity configurations are complete.

- Portrait and landscape settings are internally consistent.

An invalid configuration should not be used silently.

# 36. Documentation Principle

The repository shall contain current documentation describing:

- Requirements.

- Methodology.

- Active parameters.

- Architecture.

- Permissions.

- Data model.

- Testing.

- Build process.

- Known limitations.

- Open questions.

- Technical handover.

Every material change to the methodology should update the relevant documentation.

The code and documents must not describe different systems.

# 37. Requirements Traceability Principle

Each major implementation component should be traceable to:

- Functional requirement.

- Methodology chapter.

- Source document.

- Test case.

- Implementation module.

- Current status.

A requirements traceability table should include:

| Requirement ID | Description | Source | Component | Test | Status |
| --- | --- | --- | --- | --- | --- |

This helps ensure that no invention component is accidentally omitted.

# 38. Change-Control Principle

Changes to methodology must be deliberate and documented.

A methodology change should record:

- What changed.

- Why it changed.

- Who approved it.

- Which parameters or modules are affected.

- Whether historical data remains comparable.

- Whether tests were updated.

- Whether documentation was updated.

- Which build first contains the change.

A UI change that does not affect methodology may follow a lighter process.

A change to angle, multiplier, cumulative load, recovery, score or alert logic requires formal documentation.

# 39. Future Extensibility Principle

The Android MVP should not include unnecessary future features, but its architecture should allow later expansion.

Future additions may include:

- Cloud synchronisation.

- Parent dashboard.

- School dashboard.

- Research portal.

- Clinician or public-health view.

- Multi-user deployment.

- iOS implementation.

- AI-assisted behavioural adaptation.

- Population-level analytics.

The filed invention anticipates institutional views for schools, parents and health professionals.

The MVP should therefore use clear data models and stable interfaces without prematurely building the complete institutional platform.

# 40. Engineering Definition of Integrity

An implementation shall be considered methodologically faithful only if:

- Objective angle measurement remains independent of calibration and behaviour.

- Angle is converted through an explicit non-linear load mapping.

- Activity classification is separate and evidence-based.

- Activity multipliers are applied only in the Behaviour Domain.

- Exposure accumulates over valid monitoring intervals.

- Recovery responds to genuine sustained correction.

- Score derives from cumulative behavioural state.

- Ms Angle Angel is driven by the approved alert state.

- Dismissal is not confused with correction.

- All important values are configurable and documented.

- Historical data remains separate from the live state.

- No unsupported clinical claims or hidden formulas are introduced.

# 41. Guiding Engineering Principle

Build PARALLEYE as a transparent chain of independently testable components. Preserve objective measurement, apply behavioural interpretation only in the appropriate layer, maintain a single authoritative state for every output, and ensure every threshold, multiplier, timer and formula is visible, configurable and traceable.

# 42. Chapter Summary

The Android implementation shall preserve clear boundaries between:

- Sensor acquisition.

- Signal processing.

- Device-angle calculation.

- Neck-flexion estimation.

- Angle-load mapping.

- Activity classification.

- Engagement context.

- Activity multiplier.

- Dynamic-load calculation.

- Cumulative-load accumulation.

- Recovery.

- Score normalisation.

- Responsiveness adjustment.

- Alert-state determination.

- User-facing presentation.

- Session and historical reporting.

The implementation must not conceal the methodology inside a single opaque score function.

Each component must have:

- A defined input.

- A defined output.

- One authoritative owner.

- Centralised configuration.

- Documentation.

- Independent tests.

- Traceability to the approved PARALLEYE methodology.

This engineering structure shall guide the detailed sensor framework and calculation methodology defined in the following chapters.