# PARALLEYE Technical Methodology

## Chapter 3 – Sensor Framework, Signal Processing and Orientation Measurement

Document: PARALLEYE Technical Methodology
Chapter: 3 – Sensor Framework, Signal Processing and Orientation Measurement
Version: 1.0
Status: Draft for Approval
Project: PARALLEYE Android MVP

# 1. Purpose of This Chapter

This chapter defines the sensor framework used by PARALLEYE to acquire, stabilise and interpret device-orientation data.

It establishes how the Android application should:

- Access the required motion sensors.

- Process accelerometer and gyroscope data.

- Maintain a stable device-orientation estimate.

- Calculate the objective live device angle.

- Distinguish device angle from estimated neck flexion.

- Handle portrait and landscape use.

- Manage screen rotation.

- Reject brief or irrelevant movement.

- Maintain consistency across Android devices.

- Detect invalid or unavailable sensor information.

This chapter concerns the Measurement Domain only.

It does not define:

- Activity multipliers.

- Cumulative-load accumulation.

- Recovery.

- Score calculation.

- Alert thresholds.

- Calibration-based behavioural adaptation.

Those functions are addressed in later chapters.

# 2. Measurement Objective

The immediate engineering objective is to determine the orientation of the smartphone relative to a defined reference direction while the user is actively using the device.

The source documents describe the use of:

- Accelerometer.

- Gyroscope.

- Sensor fusion.

- A minimum initial sampling rate of 50 Hz.

The filed invention similarly identifies accelerometer, gyroscope and sensor fusion using a complementary filter or Kalman filter as possible approaches.

The Measurement Domain shall produce objective outputs suitable for later behavioural processing.

At minimum, it should provide:

- Raw sensor values.

- Filtered orientation values.

- Screen orientation.

- Validity state.

- Device angle.

- Estimated neck-flexion angle, where defined.

- Timestamp.

- Confidence or quality indicator where implemented.

# 3. Sensor Inputs

## 3.1 Accelerometer

The accelerometer measures acceleration along the device’s three physical axes.

The initial angle formula described in the source documents uses:

- X-axis acceleration.

- Y-axis acceleration.

- Z-axis acceleration.

The magnitude of the acceleration vector is:

√(X² + Y² + Z²)

The accelerometer provides a gravity-based reference when the phone is stationary or moving slowly.

It is useful for determining device inclination relative to gravity.

However, accelerometer readings may also contain:

- User movement.

- Taps.

- Vibration.

- Walking-related motion.

- Vehicle motion.

- Gaming movement.

- Sudden repositioning.

- Sensor noise.

The accelerometer must therefore not be treated as perfectly stable in every sensor frame.

## 3.2 Gyroscope

The gyroscope measures angular velocity around the device axes.

It is useful for:

- Detecting short-term rotation.

- Stabilising orientation estimates during movement.

- Following smooth device transitions.

- Reducing dependence on noisy instantaneous accelerometer readings.

Gyroscope data may be affected by:

- Drift over time.

- Device-specific bias.

- Sampling irregularity.

- Missing sensor support on some devices.

Gyroscope data should support the orientation estimate but should not be integrated indefinitely without correction from an absolute reference such as gravity.

## 3.3 Rotation or Orientation Sensors

Where supported and appropriate, Android-derived rotation-vector or orientation sensors may be used as part of the sensor-fusion implementation.

Such sensors may provide a system-generated fusion of:

- Accelerometer.

- Gyroscope.

- Magnetometer, where used by the platform.

The implementation must document whether it uses:

- Raw accelerometer and gyroscope inputs.

- Android rotation-vector output.

- Game rotation vector.

- A custom complementary filter.

- A Kalman filter.

- Another approved fusion method.

The application should not simultaneously use several conflicting orientation estimates without a defined authority.

## 3.4 Screen Orientation

The system must also identify the current display orientation.

At minimum:

- Portrait.

- Landscape left.

- Landscape right.

Where required:

- Reverse portrait.

Screen orientation is necessary because the physical meaning of the device axes changes relative to the user interface when the phone rotates.

Screen orientation must not be inferred only from current sensor values where Android provides a reliable display-rotation state.

## 3.5 System Timestamps

Every sensor event must be associated with a reliable monotonic timestamp.

Timestamps are required for:

- Sample ordering.

- Detecting dropped events.

- Calculating update intervals.

- Normalising accumulation later.

- Filtering.

- Transient detection.

- Session timing.

- Replay testing.

Wall-clock time should not be used for high-frequency sensor interval calculations because it may change if the device clock is adjusted.

# 4. Sensor Availability Assessment

Before monitoring begins, PARALLEYE should determine which relevant sensors are available.

The application should record:

- Accelerometer available or unavailable.

- Gyroscope available or unavailable.

- Rotation-vector sensor available or unavailable.

- Maximum and practical sampling rates.

- Sensor reporting mode.

- Relevant device limitations.

Monitoring should not begin until the minimum required sensor capability has been confirmed.

If the gyroscope is unavailable but the accelerometer is present, the application may use an approved fallback mode if such a mode is defined.

The fallback must:

- Be explicitly identified.

- Use separate configuration if required.

- Report reduced measurement capability.

- Avoid pretending that full sensor fusion remains active.

If the accelerometer is unavailable, the core angle methodology cannot operate as described and monitoring should not continue.

# 5. Sampling Framework

## 5.1 Initial Target Sampling Rate

The algorithm specification identifies a minimum sampling rate of 50 Hz.

For the MVP, 50 Hz may be treated as the initial target sensor-processing rate, subject to:

- Android hardware support.

- Battery testing.

- Background-processing restrictions.

- Pilot validation.

- Stability requirements.

The actual achieved sampling rate should be measured rather than assumed.

## 5.2 Sensor Callback Rate and Processing Rate

The rate at which Android delivers sensor events may not exactly equal the requested rate.

The system should distinguish:

- Requested sampling rate.

- Actual sensor-event rate.

- Orientation-calculation rate.

- Behaviour-engine update rate.

- UI-refresh rate.

- Database-write rate.

These do not need to be identical.

For example:

- Sensors may be processed at approximately 50 Hz.

- Behaviour calculations may run at a normalised interval.

- The live UI may update several times per second.

- Historical aggregates may be stored less frequently.

This separation supports battery efficiency without sacrificing measurement fidelity.

## 5.3 Timestamp-Normalised Processing

The application must not assume that each callback represents the same duration.

For each valid update, the system should calculate the elapsed interval from monotonic sensor timestamps.

The implementation should detect:

- Extremely short duplicate intervals.

- Long gaps.

- Out-of-order timestamps.

- Sensor pauses.

- Restart-related timestamp discontinuities.

Updates with invalid timing should not contribute to cumulative behavioural calculations.

## 5.4 High-Frequency Data and Storage

Raw high-frequency sensor data should not automatically be stored permanently.

Permanent storage of every 50 Hz reading would:

- Increase database size.

- Consume battery.

- Increase write operations.

- Create unnecessary privacy and processing burdens.

The application may retain:

- Short diagnostic buffers.

- Aggregated measurements.

- Summary values.

- Test-mode recordings.

- Limited replay datasets.

Raw sensor recording should be a development or research function, not the default production behaviour, unless separately approved.

# 6. Coordinate Systems

The implementation must clearly distinguish among:

- Device coordinate system.

- Screen coordinate system.

- Gravity reference.

- User-facing portrait orientation.

- Landscape-left orientation.

- Landscape-right orientation.

- Any transformed posture coordinate system.

Android device axes are physically fixed to the handset.

When the display rotates, the physical sensor axes do not automatically become new axes.

Therefore, orientation-aware transformation is required before angle interpretation.

The code must not use a fixed Z-axis formula blindly across all display orientations without confirming that the same physical quantity is being measured.

# 7. Objective Device-Angle Formula

The source documents provide the following formula:

θ = arccos(Z / √(X² + Y² + Z²))

The filed invention states that:

- The phone held vertically corresponds to 0°.

- The angle increases as the phone tilts backward toward horizontal.

- The increasing device angle approximates increasing neck flexion.

The implementation must verify that the chosen axis mapping produces this intended interpretation on an actual Android device.

The formula must not be accepted solely because it produces a numerical output.

It must be validated against physical test positions.

# 8. Required Angle Test Positions

The device-angle implementation should be validated using controlled physical positions.

At minimum:

## 8.1 Vertical reference

The phone is held vertically.

Expected output:

- Approximately 0° under the intended convention.

## 8.2 Mild backward tilt

The phone is tilted slightly away from vertical.

Expected output:

- Angle increases smoothly.

## 8.3 Intermediate tilt

The phone is tilted approximately halfway toward horizontal.

Expected output:

- Angle reflects the intermediate position.

## 8.4 Near-horizontal position

The phone is held close to horizontal.

Expected output:

- Angle approaches the upper part of the expected range.

## 8.5 Return to vertical

The phone returns to the vertical position.

Expected output:

- Angle returns smoothly toward 0° without excessive lag or persistent offset.

These tests must be repeated in:

- Portrait.

- Landscape left.

- Landscape right.

# 9. Angle Range and Clamping

The calculation must account for numerical errors.

The expression:

Z / √(X² + Y² + Z²)

should theoretically remain between −1 and +1.

Due to floating-point or sensor error, it may slightly exceed that range.

Before applying arccos, the ratio should be clamped to:

[-1, +1]

The implementation must also handle:

- Near-zero vector magnitude.

- NaN.

- Infinite values.

- Missing samples.

- Invalid sensor states.

An invalid sample should be rejected rather than converted into an arbitrary angle.

# 10. Signed and Unsigned Angle

The stated arccos formula produces an unsigned angular result.

It does not, by itself, distinguish between different directions of tilt that may produce the same cosine relationship.

The implementation must determine whether PARALLEYE requires:

- Only the magnitude of backward tilt.

- Direction-aware pitch.

- Detection of forward versus backward phone rotation.

- Exclusion of physically irrelevant orientations.

If directional information is required, it should be derived through an orientation representation capable of preserving sign, such as:

- Rotation matrix.

- Quaternion.

- Signed pitch calculation.

- atan2-based method.

The signed or unsigned design decision must be explicitly documented.

It must not be left accidental.

# 11. Sensor Fusion

## 11.1 Purpose

Sensor fusion should provide a more stable orientation estimate than either raw accelerometer or gyroscope input alone.

The source documents require sensor fusion to improve stability and reduce noise.

## 11.2 Accelerometer Role

The accelerometer provides a gravity-based long-term orientation reference when external movement is limited.

Its strengths include:

- Absolute relationship to gravity.

- No long-term angular drift under static conditions.

Its limitations include:

- Sensitivity to linear movement.

- Sensitivity to tapping and vibration.

- Instability during rapid movement.

## 11.3 Gyroscope Role

The gyroscope provides smooth short-term rotational information.

Its strengths include:

- Fast response.

- Good tracking during movement.

- Reduced sensitivity to linear acceleration.

Its limitations include:

- Drift.

- Bias.

- Need for periodic correction.

## 11.4 Complementary Filter

A complementary filter may combine:

- Short-term gyroscope orientation.

- Long-term accelerometer correction.

A general conceptual form is:

Fused angle =

α × (previous fused angle + gyroscope change)

+

(1 − α) × accelerometer angle

The exact coefficient must remain configurable and be selected through testing.

This formula is an implementation option, not a value specified in the uploaded documents.

If used, it must be identified as an engineering choice rather than a filed methodology constant.

## 11.5 Kalman Filter

A Kalman filter may also be used to estimate orientation while accounting for:

- Sensor uncertainty.

- Process noise.

- Measurement noise.

The filed invention mentions Kalman filtering as a possible sensor-fusion method but does not prescribe a specific model or coefficient set.

A Kalman filter should be used only if:

- Its implementation is understood.

- It is tested.

- Its parameters are documented.

- It materially improves stability.

- It does not introduce unacceptable latency.

For the MVP, a simpler stable method may be preferable if it meets the requirements.

# 12. Filtering and Smoothing

The live angle should be sufficiently stable for behavioural use.

However, excessive smoothing can delay detection and correction.

The filtering strategy must balance:

- Stability.

- Responsiveness.

- Battery consumption.

- Alert accuracy.

- Recovery timing.

Possible approaches include:

- Moving average.

- Exponential smoothing.

- Median filter.

- Low-pass filtering.

- Complementary filtering.

- Rotation-vector smoothing.

The chosen method must define:

- Window size or coefficient.

- Expected latency.

- Behaviour during startup.

- Behaviour after orientation change.

- Handling of missing samples.

All filter parameters must be centrally configurable.

# 13. Startup Stabilisation

Immediately after monitoring begins, the first sensor readings may be unstable.

The system should use a short startup-stabilisation period before treating the angle as valid for behavioural calculations.

During this period:

- Sensors initialise.

- Filter state is established.

- Invalid values are rejected.

- Orientation state is confirmed.

- No cumulative load should be added.

- No posture alert should appear.

The exact stabilisation duration must be configurable and tested.

This is separate from user baseline calibration.

# 14. Transient-Movement Rejection

PARALLEYE should not interpret every brief device movement as a meaningful posture change.

Examples include:

- Picking up the phone.

- Rotating the phone.

- Adjusting grip.

- Tapping strongly.

- Moving between portrait and landscape.

- Placing the phone on a table.

- Briefly lowering or raising the phone.

- Walking while holding the device.

The system should identify transient states using factors such as:

- Angular velocity.

- Acceleration magnitude.

- Orientation instability.

- Short duration.

- Screen-rotation transition.

- Lack of sustained interaction.

During a recognised transient:

- The current angle may still be measured.

- Cumulative-load accumulation may be paused.

- Recovery should not be falsely activated.

- Alert transitions should not be triggered.

- The system should resume only after stable data returns.

The transient rules must be explicit and testable.

# 15. Stable-Posture Qualification

A measured angle should be considered behaviourally valid only when the device orientation is sufficiently stable for a defined period or sequence of samples.

Stability may be based on:

- Maximum permitted angular variation.

- Maximum gyroscope magnitude.

- Maximum acceleration deviation from gravity.

- Minimum number of valid samples.

- Minimum stable duration.

A brief stable posture qualification can prevent false state transitions while remaining responsive.

The qualification should not be so long that PARALLEYE fails to detect genuine poor posture.

# 16. Portrait Orientation

In portrait mode, the application must define:

- Which device axis corresponds to the intended tilt direction.

- How the angle formula is applied.

- How left-right roll is handled.

- Whether extreme roll invalidates the posture estimate.

- How screen rotation affects the axis map.

- Where Ms Angle Angel appears.

Portrait measurement must be validated independently.

The implementation should not assume that the formula is correct merely because the user interface is in portrait mode.

# 17. Landscape Orientation

Landscape must be treated as a valid monitoring orientation.

The implementation must support:

- Landscape left.

- Landscape right.

- Axis remapping.

- Display-rotation changes.

- Orientation-specific filtering.

- Stable transition from portrait.

- Stable transition back to portrait.

The same physical phone posture should produce logically consistent exposure outputs in both landscape directions.

Landscape may use different behavioural thresholds or timing only where the approved methodology explicitly requires it.

Any landscape leniency belongs in the Behaviour Domain, not in an undocumented distortion of the measured angle.

# 18. Orientation Transition State

When the device changes between portrait and landscape, the system should enter a temporary orientation-transition state.

During the transition:

- Axis mapping is updated.

- Filter state may be reinitialised or transformed.

- Unstable measurements are rejected.

- Load accumulation pauses.

- Recovery pauses.

- Alert transitions pause.

- Existing cumulative load is preserved.

- The current mascot may remain stable or temporarily hide according to the UI specification.

Monitoring should resume after the new orientation becomes stable.

The implementation must avoid:

- Sudden 0° or 90° jumps.

- False red-zone classification.

- False recovery.

- Duplicate sessions.

- Cumulative-load spikes.

# 19. Device Motion Versus Posture

The device angle can change because:

- The user changes neck posture.

- The user moves the phone while keeping the neck posture similar.

- Both change together.

The smartphone cannot directly observe the neck without a camera or wearable sensor.

Therefore, PARALLEYE uses device orientation as a proxy.

The filed invention explicitly describes phone tilt as an approximation of neck flexion.

The application must not present the sensor output as a direct anatomical measurement without qualification.

Internally and in technical documentation, it should distinguish:

- Device angle.

- Estimated neck-flexion exposure.

- Behavioural posture state.

# 20. Device Angle and Neck-Flexion Estimate

The filed invention describes an approximate proxy relationship:

Neck Flexion ≈ (90° − Phone Angle) × 0.4–0.5

with possible adjustment for user height.

However, this relationship appears conceptually different from the separate device-angle convention where:

- Vertical phone = 0°.

- Angle increases toward horizontal.

The two descriptions must therefore not be combined automatically without confirming their reference conventions.

Before implementing a neck-flexion transformation, the technical team must define:

- What “Phone Angle” means in that equation.

- Whether it is measured from vertical or horizontal.

- Whether the transform should use θ or 90° − θ.

- Whether the factor is 0.4, 0.5 or configurable.

- Whether height affects the estimate or only behavioural personalisation.

- How the result is validated.

Until resolved, the system should preserve the objective device angle as the primary measurement and keep any anatomical estimate modular and clearly provisional.

# 21. Height and Measurement Boundary

Height is collected during onboarding.

The filed invention suggests height may influence:

- Habitual phone-holding position.

- Viewing geometry.

- Comfortable posture range.

- User-specific baseline profile.

However, the later clarification governing the current product states that calibration and profile data must not bias the objective live-angle calculation.

Therefore:

- Height may be stored.

- Height may contribute to future validated personalisation.

- Height may contribute to behavioural progression if explicitly defined.

- Height must not silently alter the measured device angle.

- Any height-based neck-flexion transformation must be separately documented and approved.

# 22. Device Placement and Unsupported Use Cases

The methodology is intended for handheld smartphone use.

The measurement may not be valid when the phone is:

- Lying flat on a table.

- Mounted in a vehicle.

- Placed on a stand.

- Being carried in a pocket.

- Being used by another person.

- Moving without active screen use.

- Positioned far from the user.

- Used as a television screen without being held.

The system should identify unsupported or low-confidence states where possible.

Possible signals include:

- Screen-off state.

- Lack of interaction.

- Device stationary in a flat orientation.

- Charging and stationary state.

- Prolonged absence of touch.

- Foreground context.

- Proximity information, if approved and available.

Unsupported states should not automatically contribute to posture load.

# 23. Active-Use Qualification

Sensor monitoring may continue technically in the background, but behavioural posture assessment should occur only during valid active use.

An active-use state may require an approved combination of:

- Screen on.

- Device unlocked.

- Recent user interaction.

- Foreground application present.

- Monitoring service active.

- Valid sensor data.

- Device not in excluded placement state.

The exact active-use rules will be finalised in the monitoring and activity chapters.

The Sensor Framework must provide the necessary system-state observations to support that decision.

# 24. Screen-Off Behaviour

When the screen is off:

- Posture load should normally not accumulate.

- Ms Angle Angel should not appear.

- Recovery should not be assumed solely because the phone angle changes.

- Sensor processing may be reduced or paused.

- Session-end or inactivity rules may be applied.

- Existing cumulative load should remain preserved according to session rules.

The application should not interpret a phone placed flat after screen-off as active healthy-posture recovery.

# 25. Sudden Invalid Angles

The system should detect physically or mathematically implausible outputs.

Examples include:

- Large angle jumps within one short interval.

- Persistent values inconsistent with gravity magnitude.

- NaN.

- Infinite output.

- Negative vector magnitude.

- Repeated identical stale values after sensor loss.

- Angle outside the permitted mathematical range.

An invalid sample should:

- Be rejected.

- Not contribute to accumulation.

- Not trigger recovery.

- Not change the mascot state.

- Be counted for diagnostics.

- Cause degraded-state handling if repeated.

# 26. Gravity-Magnitude Validation

Under relatively stable conditions, the accelerometer vector magnitude should remain reasonably close to gravitational acceleration.

Large deviation may indicate:

- Strong linear movement.

- Impact.

- Shaking.

- Vehicle motion.

- Sensor error.

The implementation may use an acceptable gravity-magnitude tolerance to determine whether accelerometer-based inclination is reliable.

The exact tolerance is an engineering parameter and must be validated.

It is not specified in the source documents.

# 27. Confidence or Quality State

The measurement pipeline should produce a quality state in addition to the angle.

A possible quality classification is:

- Valid.

- Stabilising.

- Transient movement.

- Orientation transition.

- Low confidence.

- Sensor unavailable.

- Unsupported device state.

- Invalid.

Later behavioural layers should accumulate load only when the measurement state is valid.

This is safer than forcing every sensor update into an angle and load value.

# 28. Sensor Lifecycle

The application must define when sensors are:

- Registered.

- Activated.

- Paused.

- Unregistered.

- Reinitialised.

- Resumed after restart.

Sensor lifecycle should align with:

- Monitoring service state.

- Screen state.

- User permission state.

- Battery restrictions.

- Device restart.

- Application reset.

Sensors should not remain active unnecessarily when monitoring is disabled.

# 29. Device Restart

After a device restart:

- The approved Android boot mechanism may restart the monitoring service.

- Sensor components must initialise again.

- Screen orientation must be detected.

- Filter state must be reset.

- Startup stabilisation must complete.

- No load should accumulate before valid measurement resumes.

- The stored profile and original baseline must remain available.

- Existing historical records must remain intact.

- Current-session handling must follow the defined session policy.

Auto-resume must operate only where Android permits and where the user has granted the required permissions.

# 30. Foreground Service Integration

Where Android requires a foreground service for continuous monitoring, the sensor framework should operate within that service or through a service-managed component.

The service should:

- Maintain sensor registration.

- Expose monitoring status.

- Produce a persistent notification where required.

- Recover safely from process recreation.

- Avoid duplicate sensor listeners.

- Release resources when stopped.

- Publish measurement outputs to the rest of the application.

The foreground-service notification should not expose unnecessary personal or posture details on the lock screen unless approved.

# 31. Threading and Concurrency

Sensor callbacks may arrive frequently and should not block the main UI thread.

The implementation should use a controlled processing pipeline.

It must avoid:

- Concurrent updates to cumulative state.

- Duplicate angle calculations.

- Race conditions during orientation transition.

- UI thread overload.

- Database writes from every sensor callback.

- Competing sensor listeners.

There should be one authoritative stream of processed measurement data.

# 32. Measurement Data Model

A processed measurement object may contain:

MeasurementSample

- timestamp

- rawAccelerometerX

- rawAccelerometerY

- rawAccelerometerZ

- rawGyroscopeX

- rawGyroscopeY

- rawGyroscopeZ

- screenOrientation

- filteredOrientation

- deviceAngleDegrees

- estimatedNeckFlexionDegrees

- angularVelocity

- stabilityState

- measurementQuality

- sensorMode

- configurationVersion

Not every field needs to be permanently stored.

The structure should support testing, debugging and downstream behaviour processing.

# 33. Measurement Output Contract

The Measurement Domain should provide downstream components with a clear contract.

For each valid update, it should provide:

- Timestamp.

- Device angle.

- Estimated neck flexion, if enabled.

- Screen orientation.

- Stability state.

- Quality state.

- Relevant activity-observation inputs.

- Sensor-mode indicator.

The Measurement Domain should not provide:

- Activity multiplier.

- Dynamic load.

- Cumulative load.

- Score.

- Mascot state.

Those are produced by later layers.

# 34. Configuration Parameters

The Sensor Framework should centralise at least the following configurable values:

- Requested accelerometer sampling rate.

- Requested gyroscope sampling rate.

- Processing rate.

- Filter type.

- Filter coefficient or window.

- Startup-stabilisation duration.

- Gravity-magnitude tolerance.

- Maximum valid angular velocity.

- Stable-posture qualification duration.

- Maximum accepted sample gap.

- Transient-rejection duration.

- Orientation-transition stabilisation duration.

- Angle clamping range.

- Gyroscope fallback behaviour.

- Accelerometer-only fallback behaviour.

- Debug-recording state.

- UI update interval.

All active values must be recorded with the build’s configuration version.

# 35. Testing Requirements

The Sensor Framework must support unit, simulation and physical-device testing.

## 35.1 Unit tests

Test:

- Vector magnitude.

- Ratio clamping.

- arccos conversion.

- Invalid-vector handling.

- Axis transformations.

- Portrait mapping.

- Landscape-left mapping.

- Landscape-right mapping.

- Timestamp-gap handling.

- Filter calculations.

- Stability classification.

## 35.2 Synthetic-vector tests

Use known acceleration vectors representing:

- Vertical.

- Mild tilt.

- Intermediate tilt.

- Near horizontal.

- Orientation reversal.

- Invalid zero vector.

Confirm expected angle outputs.

## 35.3 Replay tests

Use prerecorded sequences including:

- Gradual tilt.

- Sudden movement.

- Tapping vibration.

- Portrait-to-landscape transition.

- Landscape-to-portrait transition.

- Stable correction.

- Sensor interruption.

## 35.4 Physical-device tests

Test on several Android devices with different:

- Manufacturers.

- Android versions.

- Sensor hardware.

- Screen sizes.

- Refresh rates.

- Power-management behaviours.

# 36. Acceptance Criteria

The Sensor Framework shall be considered acceptable when:

- Required sensors are detected correctly.

- The measurement pipeline starts and stops reliably.

- Vertical phone position produces the approved near-zero reference.

- Angle increases consistently as the phone moves toward horizontal.

- Portrait and landscape outputs are logically consistent.

- Screen rotation does not cause large false-angle spikes.

- Sudden transient movement does not trigger sustained posture states.

- Invalid sensor data does not contribute to behavioural calculations.

- Sensor timestamps are handled correctly.

- Filter latency remains acceptable.

- Device restart reinitialises the measurement pipeline safely.

- Measurement outputs remain separate from calibration, scoring and alerts.

- The active sensor and filter configuration is documented.

# 37. Known Methodological Questions

The source documents do not fully resolve the following points:

- Whether the stated arccos formula is the final device-angle calculation for every orientation.

- Whether a signed pitch value is required.

- The exact sensor-fusion method.

- The exact filter parameters.

- How landscape axes should be transformed.

- Whether the 50 Hz rate is essential or provisional.

- The exact relationship between device angle and estimated neck flexion.

- Whether the (90° − Phone Angle) × 0.4–0.5 relationship uses the same angle convention as the device-angle formula.

- Whether height alters anatomical estimation or only behavioural personalisation.

- How unsupported device-placement states should be detected.

These questions must be documented in the project’s open-questions register.

Claude Code may implement the modular architecture and validated objective device-angle calculation while keeping unresolved transformations configurable or disabled until approved.

# 38. Guiding Sensor Principle

Acquire the device’s motion data reliably, stabilise it transparently, preserve its physical meaning, and produce one objective orientation output that remains independent of calibration, behavioural weighting, scoring and alert logic.

# 39. Chapter Summary

The PARALLEYE Sensor Framework consists of:

- Sensor availability assessment.

- Accelerometer acquisition.

- Gyroscope acquisition.

- Screen-orientation detection.

- Reliable monotonic timestamps.

- Sensor fusion.

- Signal filtering.

- Objective device-angle calculation.

- Optional modular neck-flexion estimation.

- Portrait axis handling.

- Landscape-left and landscape-right handling.

- Startup stabilisation.

- Transient-movement rejection.

- Stable-posture qualification.

- Measurement-quality classification.

- Sensor lifecycle management.

- Foreground-service integration.

- Centralised configuration.

- Unit, replay and physical-device testing.

This chapter establishes the objective Measurement Domain.

The next chapter should define Calibration and Baseline Personalisation, including what is captured during onboarding, how the baseline is stored, how it supports gradual intervention, and how it remains independent from the live angle calculation.